BAILEY PAINTING RESPIRATORY PROTECTION PROGRAM

TABLE OF CONTENTS

PURPOSE

SCOPE

RESPONSIBILITIES:

- Program Administrator

- Supervisors

- Employees

PROGRAM ELEMENTS:

- Selection Procedures

- Cartridge/Canister change schedule for vapors and gasses

- Particulate Protection

- Updating the Hazard Assessment

- Voluntary Respirator Use

- Medical Evaluation

- Fit Testing

- Respirator Use

- Cleaning & Disinfecting

- Storage

- When to Inspect Respirator

- What to Inspect on Respirator

- Defective Respirators

- Training and Information

PROGRAM EVALUATION

RECORD KEEPING

PURPOSE:

Bailey Painting has determined that employees in the work area are exposed to respiratory hazards during routine operations. These hazards include dust, paint, particulates and vapors and in some cases represent Immediately Dangerous to Life or Health (IDLH) conditions. The purpose of this program is to ensure that all Bailey Painting employees are protected from exposure to these respiratory hazards.

Engineering controls, such as ventilation and substitution of less toxic materials, are the first line of defense at Bailey Painting however, engineering controls have not always been feasible for some of our operations or have not always completely controlled the identified hazards. In these situations, respirators and other protective equipment must be used. Respirators are also needed to protect employees' health during emergencies. The work processes requiring respirator use at Bailey Painting are outlined in Table 1 of the Program Elements section.

In addition, some employees have expressed a desire to wear respirators during certain operations that do not require respiratory protection. As a general policy, Bailey Painting will review each of these requests on a case-by-case basis. As outlined in the scope section of this program, voluntary respirator use is subject to certain requirements of this program.

SCOPE:

This program applies to all employees who are required to wear respirators during normal work operations, and during some non-routine or emergency operations such as a spill of a hazardous substance. This includes employees in the work area. All employees working in these areas and engaged in certain processes or tasks (as outline in the table 1) must be enrolled in the company's respiratory protection program.

In addition, this program applies to any employee who voluntarily wears a respirator when a respirator is not required.

Employees participating in the respiratory program do so at no cost to them. The expense associated with training, medical evaluations, and respiratory protection equipment will be borne by the company.

RESPONSIBILITIES:

1. Program Administrator

The program administrator for Bailey Painting is Bruce Bailey.

The program administrator is required to oversee and evaluate the respirator program.          	This individual must be suitably trained and have the appropriate accountability and 	responsibility to manage the full respiratory protection program.

Duties of the program administrator include:

- Identifying work areas, processes or tasks that require workers to wear respirators, and evaluating hazards.

- Selection of respiratory protection options.

- Monitoring respirators use to ensure that respirators are used in accordance with their certifications.

- Arranging for and/or conduction training.

- Ensuring proper storage and maintenance of respirator protection equipment.

- Administering the medical surveillance program.

- Conducting quantitative and qualitative fit testing according to OSHA-accepted protocols.

- Maintaining records required by the program.

- Evaluating the program for effectiveness.

- Updating written program, as needed.

2. Supervisors

Supervisors are responsible for ensuring that the respiratory protection program is implemented in their particular areas. In addition to being knowledgeable about the program requirements for their own protection, supervisors must also ensure that the program is understood and followed by the employees under their charge. Duties of the 	supervisor include:

- Ensuring that employees under their supervision (including new hires) have received appropriate training, fit testing, and annual medical evaluations.

- Ensuring the availability of appropriate respirators and accessories.

- Being aware of tasks requiring the use of respirators.

- Enforcing the proper use of the respirators when necessary.

- Ensuring that respirators are properly cleaned, maintained, and stored according to the respiratory protection program.

- Ensuring that respirators do not cause discomfort.

- Continually monitoring work areas and operations to identify respiratory hazard.

- Coordinating with the program administrator on how to address respiratory hazards or other concerns regarding the program.

3. Employees

Each employee has the responsibility to wear his or her respirator when and where required and in the manner in which they were trained. Employees must also:

- Care for and maintain their respirators as instructed and store them in a clean sanitary location.

- Inform their supervisor if the respirator no longer fits well and request a new one that fits properly.

- Inform their supervisor or the program administrator of any respiratory hazards they believe are not adequately addressed in the workplace and of any other concerns they have regarding the program.

PROGRAM ELEMENTS:

1. Selection Procedures

The program administrator will select respirators to be used on site, based on the hazards to which workers are exposed and in accordance with all OSHA standards. All respirators must be certified by the National Institute for Occupational Safety and Health (NIOSH) and shall be used in accordance with the terms of that certification. Also, all filters, cartridges, and canisters must be labeled with the appropriate NIOSH approval label. The label must not be removed or defaced while it is in use.

The program administrator will conduct a hazard evaluation for each operation, process, or work area where airborne contaminants may be present in routine operations or during an emergency. The hazard evaluation will include:

- Identification and development of a list of hazardous substances used in the workplace, by department, or work process.

- Review of work processes to determine where potential exposures to these hazardous substances may occur. This review shall be conducted by surveying the workplace, reviewing process records, and talking with employees and supervisors.

- Respiratory hazards will be identified and evaluated through reasonable estimates using methods such as monitoring.

TABLE 1

VOLUNTARY AND REQUIRED RESPIRATOR USE AT BAILEY PAINTING

| Respirator | Department/Process | Hazard |
| --- | --- | --- |
| Half-face with organic filter | 1803 W. Phoenix Drive | Paint Vapors |

A sufficient number of respirator models and sizes must be available to ensure correct fit.

2. Cartridge/Canister Change Schedule for Vapors and Gases

TABLE 2

| Respirator/Cartridge | Work Area | Change Schedule |
| --- | --- | --- |

- Particulate Protection

Bailey Painting will provide an air-purifying respirator with a NIOSH certified filter for particulates.

- Updating the Hazard Assessment

The program administrator must revise and update the hazard assessment as needed (i.e., any time work processes change that may potentially affect exposure). If an employee believes that respiratory protection is needed during a particular activity, he/she is to contact his or her supervisor or the program administrator. The program administrator will evaluate the potential hazard, arranging the outside assistance as necessary. The program administer will then communicate the results of the assessment back to the employees. If it is determined that respiratory protection is necessary, all other elements of this program will be in effect for those tasks and this program will be updated accordingly.

- Voluntary Respirator Use

Bailey Painting will provide respirators at no charge to employees for voluntary use for the following work processes:

TABLE 3

| RESPIRATOR | WORK AREA |
| --- | --- |
| Half-face with organic filter | Painting |

The program administrator will provide all employees who voluntarily choose to wear either of the above respirators with a copy of Appendix D of the standard. (Appendix D details the requirements for voluntary use of respirators by employees). Employees choosing to wear a half-face piece APR must comply with the procedures for Medical Evaluation, Respirator Use, and Cleaning, Maintenance and Storage.

The program administrator shall authorize voluntary use of respiratory protective equipment as requested by all other workers on a case-by-case basis, depending on specific workplace conditions and the results of the medical evaluation.

- Medical Evaluation

Employers must provide a medical evaluation to determine each employee’s fitness to wear a respirator. The evaluation must be provided before the initial fit testing and before the respirator is used for the first time. Medical evaluations consist of the administration of a provision of a physical examination that elicits the same information as the questionnaire for the employee. An employer, who opts to provide physical examinations tohis or her employees, need not also administer the medical questionnaire. These evaluations are for those whose only respirator would be the use of escape-only respirators. Employees who refuse to be medically evaluated cannot be assigned to work in areas where they are required to wear a respirator.

A licensed physician at _________________________________, will provide the medical evaluations. Medical evaluation procedures are as follows:

- Identify physician or other licensed health care professional (PLHCP)

- To the extent feasible, the company will assist employees who are unable to read the questionnaire (by providing help in reading the questionnaire). When this is not possible, the employee will be sent directly to the physician for medical evaluation.

- The medical questionnaire and examinations will be given confidentially during the employee’s normal working hours or at a time and place convenient to the employee. The medical questionnaire will be administered in a manner that ensures that the employee understands its content.

- Follow-up medical exams will be granted to employees as required by the standard, and/or as deemed necessary by the __________________________medical clinic physician.

- All employees will be granted the opportunity to speak with the physician about their medical evaluation, if they so request.

- The program administrator has provided the ________________________ clinic physician with a copy of this program, a copy of the respiratory protection standard, the list of hazardous area or job title, proposed respirator type and weight, length of time required to wear respirator, expected physical workload (light, moderate, or heavy). Potential temperature and humidity extremes, and any additional protective clothing required.

- Written recommendation regarding the employee’s ability to use the respirator shall provide only the following information:

  - Limitations on respirator use including whether or not employee is medically able to use the respirator.

  - Need for follow up evaluation.

  - A statement that the PLHCP has provided the employee a copy of the written recommendations.

- Any employee required for medical reasons to wear a positive pressure air purifying respirator will be provided with a powered air purifying respirator.

- After an employee has received clearance and begun to wear his or her respirator, additional medical evaluations will be provided under the following circumstances.

- Employee reports signs and/or dizziness, chest pains, or wheezing.

- The ________________________________ clinic physician or supervisor informs the program administrator that the employee needs to be reevaluated.

- Information from this program, including observations made during fit testing and program evaluation, indicates a need for reevaluation.

- A change occurs in workplace conditions that may result in an increased physiological burden on the employee.

- All examinations and questionnaires are to remain confidential between the employee and the physician.

- Fit Testing

Employees who are required to wear a positive or negative tightfitting respirator must be fit tested with the same make, model, style, and size of respirator that will be used.

Employees are required to be fit tested:

- Prior to being allowed to wear any respirator with a tight fitting facepiece.

- Annually

- When there are changes in the employee’s physical condition that could affect respiratory fit (e.g., obvious change in body weight, facial scarring, dental changes etc.).

Fit tests will be qualitative fit test (QLFT) or quantitative fit test (QNFT) and will be administered according to Appendix A of 29 CFR 1910.134. The type of fit test administered will be determined by the type of respirator worn and the fit factor that is required to be achieved.

TABLE 4

ACCEPTABLE FIT-TESTING METHODS

|  | QLFT | QNFT |
| --- | --- | --- |
| Filtering facepiece | Yes | No |
| Half-Face, Negative Pressure, APR {<100 fit factor} | Yes | Yes |

The program administrator will provide an employee with an opportunity to select a different respirator and to be retested if the employee determines the fit of the respirator is unacceptable.

- Respirator Use

Employees are not permitted to wear tight-fitting respirators if they have any condition, such as fascial scars, facial hair or missing dentures, which prevents them from achieving a good seal. Employees are not permitted to wear headphones, jewelry, or other articles that may interfere with the facepiece-to-face seal. If an employee wears corrective glasses or goggles it must not interfere with the facepiece to face seal.

All employees shall conduct user seal checks each time that they wear their respirator. Employees must use a positive and negative pressure check specified in Appendix B-1 of the respiratory protection standard or respirator manufacturer’s recommended user seal check method.

The effectiveness of respirators will be reevaluated whenever there is a change in work area condition or degree of employee exposure.

All employees must leave the work area for the following reasons:

- To wash their face and respirator to prevent eye and skin irritation.

- If they detect vapor or gas breakthrough, changes in breathing resistance or leakage of the facepiece.

- To replace the respirator or the filter cartridge or canister.

NOTE: The respirator must be replaced by the employer if the employee detects vapor or gas breakthrough, changes in breathing resistance or leakage of the facepiece.

Employees will use their respirators under conditions specified by this program, and in accordance with the training they receive on the use of each particular model. In addition, the respirator shall not be used in a manner for which it is not certified by NIOSH or by its manufacturer.

- Cleaning & Disinfecting

Bailey Painting will provide a clean sanitary respirator in good working order for employee use.

Respirators are to be regularly cleaned and disinfected using procedures in Appendix B2 of 29 CRF 1910.134 or respirator manufacturer’s procedure if at least as effective as Appendix B2.

The following respirator cleaning and disinfecting schedule will be followed:

- As often as necessary to be maintained in a sanitary condition.

- Before being worn by different individuals

- After emergency sue, fit testing, and training

- Storage

Respirators will be stored as follows:

- In crew managers vehicles to protect them from damage, contamination, dust, sunlight, extreme temperatures, excessive moisture, and damaging chemicals, and they shall be packed or stored to prevent deformation of the facepiece and exhalation valve.

- In addition to above, emergency respirators will be kept accessible to the work area, clearly marked as containing emergency respirator, and stored in accordance with any applicable manufacture instructions.

- When to Inspect Respirator

- All respirators used in routine situations before each use and during cleaning.

- Emergency respirators will be inspected as lease monthly and in accordance with the manufacturer’s recommendations and shall be checked for proper function before and after each use.

- Emergency escape-only respirators shall be inspected before being carried into the workplace for use.

- Self-contained breathing apparatus (SCBA) will be inspected monthly.

- What to Inspect on Respirator

- Tightness of connections

- Facepiece: cracks, tears, or holes facemask distortion cracked or loose lenses/faceshield

- Headstraps: breaks or tears broken buckles

- Valves: residue or dirt cracks or tears in vale material

- Connecting Tube: kinks, tears, holes, brittle

- Cartridges/canisters/filters: NIOSH approval designation, gaskets, cracks or dents in housing, proper cartridge for hazard

- Elastic parts: pliability, signs of deterioration

Air and oxygen cylinders will be maintained in a fully charged state and recharged when pressure falls to 90% of manufactures recommended pressure level. Check proper function of regulator and warning device.

- Defective Respirators

Defective respirators will be moved from service and will be discarded or repaired according to the following procedure:

- Appropriate training personnel will repair or adjust respirator using only approved NIOSH approved parts designed for that respirator.

- According to manufacturer’s recommendations and specifications for type and extent of repairs to be performed.

- Reducing and admission valves regulators, and alarms will be adjusted or repaired only by the manufacturer, or a technician trained by the manufacturer.

- Training and Information

Comprehensive and understandable training will be provided to employees prior to respirator use and at least annually as follows:

- Basic information in Appendix D of 20 CFR1910.134 for voluntary respirator use in written or oral format.

- Why the respirator is necessary and how improper fit, usage, or maintenance can compromise the protective effect of the respirator.

- The limitations and capabilities of the respirator.

- How to use the respirator effectively in emergency situations, including situations in which the respirator malfunctions.

- How to inspect, put on and remove, use, and check the seals of the respirator.

- What the procedures are for maintenance and storage of the respirator.

- How to recognize medical signs and symptoms that may limit or prevent the effective use of respirators.

- The general requirements of 29 CFR 1910.134.

Upon an employee’s first entering into employment, and at lease annually thereafter, employees will also be informed of the following:

- The existence, location, and availability of any records covered by 29 CFR 1910.1020.

- The person responsible for maintaining and providing access to records.

- Each employee’s rights of access to these records.

Retraining will be administered when there are changes in the workplace, inadequacies in the employee’s knowledge, and any other situation to ensure safe respirator use.

PROGRAM EVALUATION

The Program Administrator will conduct periodic evaluations of the workplace to ensure that the provisions of this program are being implemented. The evaluations will include site inspections, air monitoring, a review of records, regular consultations with employees who sue respirators, and their supervisors.

Factors to be assessed include, but are not limited to:

- Respirator fit (including the ability to use the respirator without interfering with effective workplace performance).

- Appropriate respirator selection for the hazards to which the employee is exposed.

- Proper respirator use under the workplace conditions the employee encounters.

- Proper respirator maintenance.

Any problems identified during this assessment will be corrected and noted in an inspection log.

RECORD KEEPING

Written information regarding medical evaluations, fit testing, and the respirator program will be maintained in 1803 W. Phoenix Drive, St. George, Utah 84770.

Medical evaluations must be retained for at least the duration of employment plus 30 years.

Qualitative and quantitative fit test records will include the following:

- The name or identification of the employee tested.

- Type of fit test performed.

- Specific make, model, style, and size of respirator tested.

- Date of test.

- The pass/fail results for QLFTs or the fit factor and strip chart recording or other recording of the test results for QNFTs.

- Fit testing records will be retained until the next fit test is administered.